Chapter 4

Types of Problems in steady state mode:

- Simulation Problem

Given the process flowsheet & the best operating conditions; calculate some process outputs

- Specification Problem

Given the process flowsheet & desired outputs from the process; calculate the required operating conditions.

- Optimization Problem

Given the process flowsheet; choose the desired outputs (& required operating conditions) to maximize or minimize some objective functions

- Synthesis Problem

Given the process inputs & desired outputs; choose the best process (& required operating conditions) to minimize some objective function

Chapter 5

- Selection of the processing mode: batch vs continuous (ch 9)

- Identification of process input-output structure

- Deciding on the process recycle structure

- Identification & design of process separation system

| Batch/semi-batch | Continuous |
| --- | --- |
| Low production | High |
| Large residence time | Small |
| Manufacture of pharmaceuticals, foods, specialty chemicals, beer production | Industrial industry, oil refinement, chemical production of 5000 tonnes/y |

| Component Type | Classification |
| --- | --- |
| Reactant | Recycle |
| Reaction intermediate | Recycle |
| Reversible by-product | Recycle or Exit |
| Primary Product, valuable by-product, fuel by-product, waste by-product | Exit |

| Type | Feed | Sep. Agent | Products | Principle |
| --- | --- | --- | --- | --- |
| Evaporation | L | Heat | L+V | Volatilities |
| Flash | L | Pressure Reduction | L+V | Volatilities |
| Distillation | L/V | Heat | L+V | Volatilities |
| Absorption | G | Liquid Solvent | L+G | Preferential Solubility |
| Stripping | L | Gas Solvent | L+V | Preferential Solubility |
| Extraction | L | Immiscible Liquid | L+L | Preferential Solubility |
| Adsorption | G or L | Solid Resin | L/G+S | Adsorbate |
| Leaching | S | Solvent | L+S | Preferential Solubility |
| Magnetic Separation | Mixed S | Magnetic Field | S+S | Magnetic Attraction |
| Fluidized Bed (continuous) | G/L | Fluidized Particle | G/L+S | Adsorbate |
| Filtration | L&S | Filter Medium | L+S | Size of Solid |
| Settling | L&S | Gravity | L+S | Difference in Density |

Profit margin= (value of product)- (value of raw material)

Chapter 3

Horizontal lines are dominant

Block Flow Diagram(BFD)

Feed Prep: Compressor, Heat-Exchanger, Pumps.

Separator: Distillation, extraction, separator

Process Flow Diagram(PFD)

| C (or K) | Compressor, Turbine, or Expander | R | Reactor |
| --- | --- | --- | --- |
| E (or HX) | Heat Exchanger | T | Distillation, absorption tower |
| P | Pump | TK | Storage Tank |
| V | Vessel |

Piping & Instrumentation Diagram (P&ID)