Name: ___________________________________	Section: _______________

## Due Date: Monday June 22 by 5pm

## Marks: 100

This lab activity is to be submitted to the lab instructor printed on paper.

## Purpose:

Demonstrate successful conversion between decimal, binary, Octal, Hexadecimal and fractional number-based systems.

## Procedure:

Convert the following numbers to the desired number base.    To obtain full marks for each question

you must show your work.

Clearly indicate your answers to the questions in a box

Include the number bases in your answers (subscript)

If you are having difficulty with this assignment, ask your lab instructor for help.

Print this lab. No hand writing accepted.

Keep track of how much time you spent on this lab and include it with your results

Part A-Convert decimal to binary and go back (show you work)

Example: 1010 00112

=27 + 26 + 2 5 + 24 + 23 + 22 + 21 + 20

=128 + 64 + 32 + 16 + 8 + 4 + 2 + 1

= (128*1) + (64*0) + (32* 1) + (16*0) + (8*0) + (4*0) + (2*1) + (1*1)

= 128 + 0 + 32 + 0 + 0 + 0 + 2 + 1

= 16310		answer

Example 5410 = 0011 01102

I added two leading zeros to the left of the number to make it an 8-bit binary number, leading zeros do no change the answer in any way.

Quotient Remainder

54/2=27  	0

27/2=13  	1

13/2=6   	1

6/2=3   	0

3/2=1   	1

½=0  		1

*Read the remainder from bottom up

1-Convert the decimal number 15210 to an 8-bit binary number

152/2= 76 remainder 	0

76/2= 38 remainder	0

38/2= 19 remainder	0

19/2= 9 remainder	1

9/2= 4 remainder 	1

4/2= 2 remainder 	0

2/2= 1 remainder	0

½= 0 remainder		1

15210 = 1001 10002

2- Convert the decimal number 75 to an 8 bit binary number

75/2= 37 remainder	1

37/2= 18 remainder	1

18/2= 9 remainder	0

9/2= 4 remainder 	1

4/2= 2 remainder 	0

2/2= 1 remainder	0

1/2=0 remainder	1

7510 = 0100 10112

3-Convert the binary number 1111 10112 to its’ decimal form

1	1	1 	1	1	0	1	1

128	+64	+32	+16	+8	+0	+2	+1

1111 10112 =  25110

4-Convert the binary number 1010112 to its decimal form

1	0	1	0	1	1

32	+0	+8	+0	+2	+1

1010112= 4310

Part B-Convert decimal to octal and binary and go back (show your work)

Example: 718 =

= 81 + 80

=8 + 1

= (8*7) + (1*1)

= 56 + 1

= 5710

Example: 18910 = 2758

Quotient Remainder

189/8=23 5 ^

23/8=2     7

2/8=0       2

*read from the bottom up

Example: 42068 = 100 010 000 1102

Take each number and convert it to a 3 bit binary number

4=100, 2=010, 0=000, 6=110

Join the numbers together to get the answer

1000100001102

Example: 1101111000012=67418

Divide the binary number into groups of 3

110 =6 111=7 100=4 001=1 Substitute an octal number for each 3 bit group

5-Convert the decimal number 34510 to its octal form

345/8= 43 remainder	1

43/8= 5 remainder	3

5/8= 0 remainder	5

34510 = 5318

6-Convert the decimal number 61210 to its octal form

612/8= 76 remainder	4

76/8= 9	remainder	4

9/8= 1 remainder	1

1/9=0 remainder	1

61210 = 11448

7-Convert the octal number 5438 to its decimal form

5	4	3

(64*5)+(8*4)+(1*3)

320     + 32    +    3

5438 = 35510

8-Convert the octal number 4568 to it binary form

| 4 | 5 | 6 |
| --- | --- | --- |
| 4 2 1 | 4 2 1 | 4 2 1 |
| 1 0 0 | 1 0 1 | 1 1 0 |

= 1001011102

9-Convert the binary number 1011102 to its octal form

| 1 0 1 | 1 1 0 |
| --- | --- |
| 4 2 1 | 4 2 1 |
| 5 | 6 |

= 568

10-Convert the binary number 111102 to its octal form

| 0 1 1 | 1 1 0 |
| --- | --- |
| 4 2 1 | 4 2 1 |
| 3 | 6 |

= 368

Part C-Convert decimal to hexadecimal and binary and go back (show your work)

Example: 132410=52C16

Quotient Remainder

1324/16=82 12 ^ 12=C

82/16=5 2 |

5/16=0 5 |

*read from the bottom up

Example: 7216=11410

(7*161) + (2*160) = 112+2 =11410

Example: 1101100110112=D9B16

*Break each number into groups of 4 and convert that number into hexadecimal

1101=D 1001=9 1011=BJoin the numbers and letters together to get the answer

11-Convert the decimal number 459 to its hexadecimal form

459/16= 28 remainder	11

28/16= 12 remainder	12

12/16= 0 remainder	1

= 1CB16

12- Convert the decimal number 123 to its hexadecimal form

123/16= 7 remainder	11

16/7= 0 remainder	7

= 7B16

13-Convert the hexadecimal number DEAD to its decimal form

D		E		A		D

3		2		1		0

(13*163)+	(14*162)+	(10*161)+	(13*160)

(53248)	+	(3586)	+	(160)	+	(13)

=5700510

14-Convert the hexadecimal number 36 to its decimal form

3		6

(161*3)	+	(160*6)

48	+	6

= 5410

15-Convert the hexadecimal number 67 to its binary form

| 6 | 7 |
| --- | --- |
| 8 4 2 1 | 8 4 2 1 |
| 0 1 1 0 | 0 1 1 1 |

= 0110 01112

16-Convert the hexadecimal number 9A3D to its binary form

| 9 | A | 3 | D |
| --- | --- | --- | --- |
| 8 4 2 1 | 8 4 2 1 | 8 4 2 1 | 8 4 2 1 |
| 1 0 0 1 | 1 0 1 0 | 0 0 1 1 | 1 1 0 1 |

= 1001 1010 0011 11012

17-Convert the binary number 1111 to its hexadecimal form

1 1 1 1

8 4 2 1

(15)

= F16

18- Convert the binary number 11001 to its hexadecimal form

| 8 4 2 1 | 8 4 2 1 |
| --- | --- |
| 0 0 0 1 | 1 0 0 1 |

- 9

= 1916

Part D- Convert decimal/fractional to binary

Example 0.437510 = 0.01112

Convert the 0 to binary first 010=02

Next convert .437510 to binary by multiplying it by 2

0.4375*2=0.875

0.875*2 = 1.75

0.75 *2 = 1.5

0.5 * 2 = 1

*Read the modulus from top down

0.0111

19-Convert the decimal number 14.75 to its binary number

14

8 4 2 1

1 1 1 0

0.75*2	=1.50

0.5*2	=1

= 1110.112

20-Convert the decimal number .34574 to its binary number

0.34574*2 =0.69148

0.69148*2 =1.38296

0.38296*2 =0.76592

0.76592*2 =1.53184

0.53184*2 =1.06368

0.06368*2 =0.12736

0.12736*2 =0.25472

0.25472*2 =0.50944

0.50944*2 =1.01888

0.01888*2 =0.03776

0.03776*2 =0.07552

0.07552*2 =0.15104

0.15104*2 =0.30208

0.30208*2 =0.60416

0.60416*2 =1.20832

0.20832*2 =0.41664

0.41664*2 =0.83328

0.83328*2 =1.66656

0.66656*2 =1.33312

[…] never ending?