# CEG3155 Quiz 2 Notes

Process blocks are considered a concurrent statement (concurrent_stmts)

PB contents run in parallel with other concurrent statements

PB only allow three components

  - Signal assignments

  - If statements

  - Case statements

We only use the PB that deals with the sensitivity list (NO wait statements)

  - Sensitivity Lists list input signals to sub-circuit

We have 5 Golden Rules:

  - All signals read with PB must be in sensitivity list (signals on right side of assignment statements, if and case statements) -> will result in a ‘WARNING’ from the synthesis tool

  - The last assignment to an output signal takes precendence over ALL previous assignments -> no way to violate rule regarding multiple assignments to output signal

  - All output signals must have unconditional assignments -> causes a real problem with no warning from the synthesis tool. A FF(output = output) is added in case none of the Boolean expressions are true

  - Never use a signal on both sides of an assignment statement, it will create a combinational loop -> synthesis tool will generate a ‘WARNING’ tool

    - Ex. Tmp <= Tmp or b

  - Never assign to an output signal both inside and outside the PB  -> generates a syntax error