Network Edge is important

What belongs to what in client server peer-per

Cable network components not on midterm could be on final.

Know Cable Modem Termination System and how it looks for the final.

Wireless Access Network stuff is more likely to be on final not the midterm

Caravan analogy from chapter 1 is on da final.

MIDTERM::

Network edge is vital. Know what belongs to client server and peer-peer

Don’t focus a lot on the different types of home networks ??

Know Slide 32 Chapter 1 really well. Have that equation memorized and be able to use it well!! (ONE QUESTION HAS 3 MATH EQUATIONS TO CALCULATE) NO CALCULATOR…..

All of our numbers will be in base 10.

No physical media

Chapter 1 Slide 39!! (Math equation) When you have 2 lines it 2L/r, e.g. multiple L/R x number of lines

Queuing delay, loss

Suppose K packets arrive simultaneously to a link at which no packets are currently being transmitted or queued. Each packet is of length L and the link had a transmission rate R. What is the average queuing delay for the K packets??

For the first packet, 0 for the next packet L/R, the third packet 2L/R

0 + L/R + 2L/R + …….. + (K-1)L/R

L/R(1+2+3+4 ….. K-1)

[L/R(K(K-1)/2)]/K

[(K-1)L]/2R

Chapter 1 Slide 45

Know Circuit Switching and Packet switching (ALL SLIDES IN THAT SECTION.

Including the formula on Slide 51 (No the binomial distribution part, there will be a hint???)

How long does it take a packet of length 1,000 bytes to propagate over a link of distance 2,500 km, propagation speed 2.5*108 m/s, and transmission rate 2 Mbps? Generally, how long does it take a packet of length L to propagate over a link of distance d, propagation speed s, and a transmission rate R bps? Does this delay depend on packet length? Does this delay depend on transmission rate?

2.5 X 108m/s = 2.5x105Km/s = 2.5x102Km/msec

2500/250 = 10 milliseconds.

Generally length of time, d/s

Delay does not depend on packet length or transmission rate

Chapter 1 slide 65. If he is asking for all delays he will state that. He will likely write it as nodal delay.

Line of 1Mbps, transmission of 1Mbps. This line can handle this

Line is 1Mbps, my transmission speed to the queue is 1.5Mbps After 1 second, .5Mbps are in the queue

Chapter 1 Slide 70 (you will be asked about different delay? Something to do with the labs??)

Throughput is important. Resistance communication?? Big shared line, handshaking in the beginning????

Normal resistance 5 objects, 2 to start then 4 so 6? I don’t even know what this means

Huge shared line no resistance, 2 round trip for each so 10?

Internet protocol stack is important. Know what’s in each.

Remember the encapsulation slide

No Network Security

No network management

No internet history

What is the difference between host and end system? There isn’t one.

List six access tech. Classify each one as a home access, enterprise, wide area etc,

What is the transmission rate of the Ethernet Lans

Ethernet

CHAPTER 2

TCP and UDP (Know what uses each)

Which is client server, which is peer-peer, which is hybrid (SKYPE is hybrid)

Skype port 1024 and up or 80-443

Port 6881-6889 and 6969 for the tracker port (BIT TORRENT)

Ten net is 23

DNS is UDP and port 53

Port 20, 21 FTP

Caching is important

Slide that is very important:

Chapter 2 Slide 20

SUPER IMPORTANT FROM ASSIGNMENT 1:

Server has to know the client IP address

Slide 128 from Chapter 2

Client address is returned TCP will not return the client address