OM assignment #5

1a)

Sample 1 = 8/300 = 0.02666 = 2.7%

Sample 2 = 9/300 = 0.03 = 3%

Sample 3 = 4/300 = 0.01333 = 1.3%

Sample 4 = 7/300 = 0.02333 = 2.3%

Sample 5 = 9/300 = 3%

Sample 6 = 6/300 = 0.02 = 2%

Sample 7 = 5/300 = 0.01666 = 1.7%

b)  The best estimate for the true percentage would be to obtain the average among all the samples.

Average = 0.02285 = 2.3%

c)  This process would be statistically in control because all the percentages of defects are within the range of 4.7% and 0.3%.  It is also randomly distributed above and below the average so there are no patterns present.  It would only be out of control if there is a sample present with a value that is higher than 4.7% and under 0.3%.

d)  p2bar = 2%

0.02(+/-) 1.96*sqrt((0.02(1-0.02)/7)

0.02 +/- 1.96*0.052915

0.02 +/- 0.1037

[-0.0837, 0.1237]  [0, 0.1237]

The confidence level at 95% would be 0% and 12.37% because it is not possible to have a negative percentage value for this process.

LCL

Mean

UCL

e)

It is out of control because all the values are above the average line and none are below.  Since there is a pattern the process is statistically out of control.

2

| Sample | X1 | X2 | X3 | X4 | X5 | X6 | mean | R |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 85.2 | 84.2 | 84.1 | 84.2 | 85.7 | 84 | 84.56666667 | 1.7 |
| 2 | 83.4 | 83.3 | 84.4 | 84.7 | 86.8 | 86.5 | 84.85 | 3.5 |
| 3 | 86.3 | 84.3 | 86.2 | 83.2 | 84.1 | 84.4 | 84.75 | 3.1 |
| 4 | 86.8 | 84.8 | 83.5 | 83.1 | 86.8 | 83.3 | 84.71666667 | 3.7 |
| 5 | 86.7 | 86.8 | 86 | 83.4 | 83.6 | 84.5 | 85.16666667 | 3.4 |
| 6 | 85.4 | 86.8 | 85.4 | 84.2 | 85.1 | 85.5 | 85.4 | 2.6 |
| 7 | 85.1 | 83.3 | 84.3 | 84.8 | 84.5 | 86 | 84.66666667 | 2.7 |
| 8 | 86.3 | 84.6 | 85 | 84.8 | 85.5 | 84.9 | 85.18333333 | 1.7 |
| 9 | 85.7 | 85.3 | 86.5 | 86.3 | 84.8 | 86.6 | 85.86666667 | 1.8 |
| 10 | 83.9 | 84.2 | 85.8 | 86.9 | 83.7 | 84.9 | 84.9 | 3.2 |
|  |  |  |  |  |  |  | X2bar | Rbar |
|  |  |  |  |  |  | average | 85.00666667 | 2.74 |
| x-chart |  |  |  |  |  | rounded | 85 | 2.74 |
| n = 6 |  |  |  |  |  |  |  |  |
| A2 = 0.483 |  |  |  |  |  |  |  |  |
| 85 +/- 0.483*2.74 |  |  |  |  |  |  |  |  |
| 85 +/- 1.32342 |  |  |  |  |  |  |  |  |
| UCL = 86.32 |  |  |  |  |  |  |  |  |
| LCL = 83.68 |  |  |  |  |  |  |  |  |
| r-chart |  |  |  |  |  |  |  |  |
| D3= 0.000 |  |  |  |  |  |  |  |  |
| D4= 2.004 |  |  |  |  |  |  |  |  |
| LCL = 2.74*0 = 0 |  |  |  |  |  |  |  |  |
| UCL = 2.74*2.004 = 5.49 |  |  |  |  |  |  |  |  |

LCL

Mean

UCL

LCL

Mean

UCL

Based on the charts, the process is in control because there aren’t any immediate identifiable patterns that would suggest that the process is out of control.

3)

UCL = 87

LCL = 84.5

X2bar = 85

K=2.536

Rbar = 2.74

Sigma = Rbar / K = 2.74/2.536 = 1.08

Z(UCL) = 2/1.08 = 1.85   0.4678

Z(LCL) = 0.5 / 1.08 = 0.46  0.1772

P = 0.4678 + 0.1772 = 0.645

Therefore, the percentage of concrete slabs that meet the designer specification is 64.5%.