CP216 – Introduction to Microprocessors

Department of Physics and Computer Science

Midterm Exam

Thursday, March 1, 2018

- Answer the following questions: 					                    	 (10 marks)

- What is an ALU		 – Arithmetic and Logic Unit

- What is a CPU		 – Central Processing Unit

- What is RISC 		– Reduce Instruction Set Computer

- What is CISC 		– Complex Instruction Set Computer

- What is RAM		 – Radom Access Memory

- Describe static RAM		 – RAM built using Transistors, no refresh required

- Describe dynamic RAM	 – RAM build using capacitors, requires refreshing

- What is ASCII  		-- American Standard Code for Information Interchange

- A memory component spans the address range 0x00400000 to 0x007FFFFF, what is its capacity?

222MB = 4MB

- Do the following Digital Logic and Arithmetic Operations:			       	   (10marks)

- Bitwise X-OR of 1100 and 0011                           		  --- 1111

- Find the 2`s complement of 11110000               		 ----  00010000

- Multiple the binary number of 11100011 and 00011100   	 ----   01100011010100

- Convert 1234.125 into 32-bit IEEE floating-point format 				          	   (10 marks)

- What is the decimal equivalent of the 32-bit IEEE floating-point value CC4C000016	   (10marks)

- Consider the following circuit that takes a 4-bit binary input and encodes it. Construct a truth table for the 16 inputs 0000 to 1111 and examine the outputs code. What is the characteristic feature of this code. Note that input b4 is always 0. 					                                                     (10marks)

- In the ARM processor, if r1 = 11110000111000101010000011111101 and

r2 = 00000000111111110000111100001111, what is the value of r3 after executing BIC r3, r1, r2

(10marks)

- If r1 = 00FF16 , and r2 = 4, what is the value of r0 after each of the following instructions has been executed?  (assume that each instruction uses the same data)

- ADD r0, r1, r1, LSL #2

- ADD r0, r1, r1, LSL #4								         (10 marks)

- What does the following code do?

Label_1  ADD r0, r1, r2       	;Add register r1 and register r2, put the result into register r0

SUBS r7, #1		;Subtract r7 by 1 (decrease r7 by 1)

BEQ Label_1		;Branch to (go to) Label_1, if r7 is not Zero (Branch to Label_1 is Z is 0)

(10 marks)

- Write an ARM assembly code to do:

IF (X == Y)

THEN Y = Y +1;

ELSE  Y = Y + 2

(20 marks)