The confocal Microscope

Only light focused at the pinhole will enter the detector

This allows the confocal to provide clear images a few um into tissue

In addition to the x and y axis, imaging may occur in the z axis

3D reconstructions possible

Uses principles of fluorescence

Doesn’t show the light that is out of focus

Don’t directly see the specimen, the computer receives the signals from detector and presents it as an image

Two-Photon Microscopy (non-linear)

Advantages:

Non-linear technique that uses higher-order light-matter interactions from multiple photons to generate contrast (needs more than 1 photon to excite the material)

Allows deep tissue imaging (up to 1 mm depth possible)

In this process, absorption occurs in the near IR region, and NIR light penetrates deep into tissue

IR and NIR can travel deeper into tissue since less interaction with matter

Disadvantage:

Very costly

Two-photon absorption involves “simultaneous (~0.5fs) arrival at excitable molecule

Excitation and emission occur, as in fluorescence

However, signal is dependent upon photon density, so absorption is spatially confined

only exciting a small point

In confocal, single photon absorption occurs throughout excitation light cone

- More energy is required

- For more than 1 photon to excite the fluorescent molecule, need to specifically focus it

- Only focusing the light on a very specific point will yield enough energy to emit a signal

Research Techniques II: Microscopy and Cell Imaging

Seeing at the cellular and subcellular level (continued)

SEM = looking at the topological features of cells

Both TEM and SEM send electrons rather than light at a specimen

Elements like gold reflect electrons,