Input hardware

- Devices that translate data into a form the computer can process.

- Translate words, numbers, sounds and pictures into binary 0s and 1s (off or on electrical signals or light pulses)

Output hardware

- Devices that translate information processed by the computer into form humans can understand.

- Translates binary code into words, numbers and pictures

Three major input hardware are keyboards, pointing devices (mouse) and source-data entry devices (scanners)

Keyboards convert characters into electrical signs

Keyboards differ depending on the language

How they work:

- You press a key

- The current flowing through the circuits is interrupted

- Processor determines where the break occurs

- It compares the location of the break with the (x,y) character map for the language on the keyboard’s ROM chip

- A character is stored in keyboard memory

- This gets sent to PC as data stream via wire or wireless connection

- OS interprets its own operating-system-specific commands and sends the others to the application for interpretation

- Most keyboards are QWERTy

- Keyboards are either tactile (psysical) or touch screen (virtual)

Types of keyboards:

- 104-108 keys desktop standard

- 80-85 keys for laptops

- Wired (connect to CPU via a serial or USB port

- Wireless: IR (infrared), RF (radio frequency) Bluetooth

Terminal types

- Dumb Terminals:

  - AKA : Video Display Terminal (VDT)

  - Has display screen and keyboard

  - Can do input and output only- no data processing

- Intelligent terminals

  - Has screen, processor, keyboard and memory

  - Can perform some independent functions

  - Automated teller machine; point-of-sale terminal; mobile data terminal

Pointing devices control the position of the cursor or pointer on the screen and allow the user to select options displayed on the screen.

- Mouse is the principal pointing device

  - Mechanical mouse has a ball that rolls when the mouse touches the desktop surface

  - Optical mouse uses laser

  - It controls the mouse pointer on the screen

  - When the cursor changes for an I-beam means text can be entered

  - The mouse has one to give buttons used for various functions (clicking, dragging etc)

- Trackball

  - Movable ball mounted on top of a stationary device

  - Good for locations where mouse can’t be moved around.

- Touchpad

  - Used by sliding the finger over the small flat surface

  - Click by tapping you finger on the surface

  - May require more practice to use than a mouse

  - Used on laptops

- Pointing stick

  - Located between the keyboard, pointing stick is a pressure sensitive device that allows the user to control the pointer with one finger

- Touch screens

- Multitouch Screens

  - Touch screens that allow more than one fingers or other gestures

- Pen Input

  - Uses stylus.

  - Uses handwriting recognition

- Light pen

  - Light sensitive-pen like device that used wired connection to a computer terminal

  - Used by graphical artist and engineers.

- Digitizer

  - Uses an electronic pen or puck to convert drawings and photos to digital data

  - Digitizing tablets are often used in architecture

- Digital pen

  - Writing instruments, can write on paper

  - Tiny camera in the pen tip captures writing

  - A microchip in the pen converts it into digital ink

  - The writing is sent as an image to the computer. (may require special paper)