Hard Drives

- The hard drive is the main storage device for all modern computers

- Storage capacity

  - Originally 5MB  Now up to 1TB

- Data transfer rates

  - Originally 85Kbps  Now as fast as 79Mbps

- Seek time

  - First hard drives around 85ms

  - Now down to 3.2ms

  - Over 26 times faster

- Cost

  - $2,000 for 10MB  Now $95 for 1T

- Some major internal components of a hard drive

  - Platters

  - Recording media

  - Read/write heads

  - Actuator arms

  - Spindle

  - Motors

  - Air Filters

- Platters

  - Hard drives use rugged and solid platters to store data

  - Platters were traditionally made of aluminum and coated with a magnetic medium

  - Newer platters are now made from a ceramic composite material

  - These are lighter and have very low thermal expansion

- Actuator Arm

  - The read/write heads are located on the tip of the actuator arm

  - The actuator arm is moved by one of two types of motors

- Spindle

  - Most hard drives have several platters separated by spacers and clamped to a rotating spindle

  - The spindle, and the platters, are rotated at a constant speed

- Recording Media

  - All platters must be covered with a thin layer of a magnetically retentive substance

  - Oxide media

    - Soft, cheap and not capable of store large amount of data

  - Thin-film media

    - Thinner, harder and more perfectly formed

    - Very smooth, allowing heads to float closer

  - AFC media

    - Called antiferromagnetically coupled media

- Read/write heads

  - Both sides of each platter is coated to provide separate magnetic surfaces

  - Every platter requires two read/write heads, one for each side

  - The read/write heads are spring loaded and will make contact with the platters at rest

  - They do not make contact with the platter during use, they float on a very thin cushion of air

  - The air buffer is less than 15 microns, by comparison a finger print is 30 microns

- Motors

  - Stepper motor

    - The first technology that moved the actuator arm in fixed increments

    - Positioning of the arm became less precise over time

    - Hard drives needed to be parked when moved

- Voice coil

  - All hard drives today use a linear motor, or voice coil

  - When electrical current is applied a magnetic field is generated that moves the actuator arm

  - Much faster, accurate and more reliable than the stepper motor

  - Hard drive heads are automatically parked

- Air Filters

  - Most hard drives have two air filters

  - Filters are permanently sealed inside the drive

  - Air is not circulated from inside to outside the drive

  - Recirculating filter

    - Filters the small particles scraped off the platters

  - Barometric filter

    - Provides venting to enable pressure equalization

- The first hard disks spun at a rate of 3,600rpm

  - This is ten times faster than a floppy diskette

- Most hard drives today spin at a rate of either 5,400rpm or 7,200rpm

- The fastest drives spin at speeds up to 15,000rpm

HARD DRIVE ORGANIZATION

- The hard drive can be divided into the following areas:

  - Tracks

  - Cylinders

  - Sectors

  - Clusters

- Tracks

  - Each platter consists of multiple concentric circles called tracks

  - A typical platter contains thousands of tracks                Tracks

- Cylinders

  - Each track is located directly over the same track on subsequent platters

  - Each group of tracks of the same diameter is called a cylinder

  - The number of tracks on one side of a platter is therefore equal to the number of cylinders

- Sectors

  - If you cut your hard drive like you would a pie, each piece or wedge would represent a sector

  - A sector is the smallest unit of space on your hard drive that can be allocated

  - Stores 512 bytes of data

- Cluster

  - A cluster is a group of sectors ranging from 2 to 64 depending on how the drive is formatted

  - A cluster is the smallest amount of space that can be used for any read/write operation

PASSWORDS

- The Role of Passwords

  - To limit computer or system access to only those who know a sequence of keyboard characters

- Breaking into a Computer without a Password

  - Trying all possible passwords algorithmically would eventually find correct password, but software usually limits the number of tries

- Forgetting a Password

  - Passwords are scrambled or encrypted and stored, so system administrator usually can't tell you your password if you forget it

- It's not a good idea to choose something easily guessed, but should be easy for you to remember

- Should have at least 8 characters, with a mix of uppercase and lowercase letters, numbers, punctuation characters

- Use a sequence not found in dictionaries

- No personal associations (like your name)

- Select a personally interesting topic or theme

  - Favorite movie, travel destination, sport/hobby

  - Always select passwords related to topic

- Make password from a phrase, not a single word

- Encode the password phrase

  - Make it short by abbreviating, replace letters and syllables with alternate characters or spellings, punctuation patterns

- Theme is Alma Mater… Oxford University

  - OxfordU    (shorten)

  - Ox4dU       (replace for with 4)

  - Ohx4dyoU   (replace O with Oh, U with yoU)

- Theme is favorite movie… Gone with the Wind

  - GWTW     (shorten)

  - G2uT2U     (replace W with 2u and 2U)

  - G2uTdosU   (replace 2 with spanish “dos”)

- A strong password:

  - At least 8 characters long

  - Contains the following characters

    - Uppercase letters

    - Lowercase letters

    - Numbers

    - Symbols found on the keyboard such as:

      - ~,!,@#,%,+, ?,/

change every 3-4 months

PHISING

- Main aim is some form of identity theft

- Spam emails are made to look like they come from trusted sources like banks, eBay, PayPal, government, etc.

  - Look very authentic, use company logos and graphics, mimic corporate web pages

- Message text will claim some problem has arisen and that you must log on to resolve the issue

SPEAR PHISHING

- Is an e-mail spoofing fraud attempt that targets a specific organization seeking confidential data for financial gain, trade secrets, or military information

- Appears to come from a trusted source i.e. someone in authority within recipient's own company

- E-mail looks authentic

- Asks you to log into a bogus page that requests user name and password or

- Click on a link to download spyware or other program.