Question Answers (MECH 368)

- What is the most important technological development in 20th century:

Electrification

- Equilibrium state:

there are always electrons moving inside a material.

- Steady State:

With a charge applied across electrons, the material is in a non-equilibrium (electrons move to positive charge)

- What is an ampere?

1A = 1C/s (A current of one ampere is one coulomb of charge going past a given point per second)

- Current: always flows opposite to electrons (positive to negative in circuit)

- Branch = portion of circuit with a component between its terminals

- Node = Junction of more than 1 branch

- Loop = Any closed connection of branches

- Mesh = any loop that doesn’t contain any other loops

- Electrical Energy = Basically potential or kinetic energy (energy due to an objects motion, before it is converted to another form of energy)

- Electrical Power = rate, per unit time, at which electrical energy is transferred by an electric circuit.

- kWh = measure of how much energy you’re using. How much energy you would use if you kept 1000 watts appliance on. (ex. 100 watt bulb = 10 hours to get to 1kWh energy/2000 watt appliance uses 1kWh in 30mins. 50 watt item could stay on for 20 hours before it used 1kWh.

- Ground = reference voltage. (usually assigned as 0V)

- How batteries work:

a chemical reaction generates the potential difference. Once the chemical reaction has “finished”, the battery no longer generates a useful potential difference and the battery is thrown away. (providing current and potential diff in rechargeable batteries, chemical rxn store this energy and use it later.

- Temp effect on batteries:

Self discharge is lowest when cold, worst when hot. So cool storage is good for a battery.

- Battery dangers:

battery can overheat, causing the cells to break open and result in a chain reaction of other cells rupturing. This is called thermal runaway.

- NI-Metal vs Li-Ion:

Ni  less expensive, larger and heavier, can both hold similar power (But LI-ion can be charged/discharged faster), more durable.

- Types of batteries:

Foldable: 200,000 cycles

Aluminium: (1,100 mile drive)

Sodium-ion :(seven times more efficient)

Carbon-ion: (superfast-charging)

Gold nanowires: (200,000 cycles)

Laser-made microsupercapacitors: (50 times faster charging)

- AWG logic: larger the size, smaller the diameter)

- Open circuit = No current, Yes voltage difference.

- Short circuit = Yes current, Always 0V (For any current)

- Practical voltage source vs. real voltage source:

Practical = no resistor in voltage source

Real = resistance inside the voltage source (some current losses)

- Practical current source vs. real current source:

Ideal = infinite internal resistance

Practical = has some internal resistance in parallel and the amount of flow depends upon the load.

- Ammeter = device that when connected in series with a circuit element, can measure the current flowing through the element

- Voltmeter = measures voltage difference across a circuit element. (must be connected in parallel) (ideal has infinite internal resistance so doesn’t draw current)

- Ohmmeter = Measures resistance of a circuit element (element must be disconnected from any other circuit)

- Example of 5 materials in an electronic circuit:

Copper, tin, paper impregnated with resin, gold, black resin

Resistors:

- Factors influencing the resistance in a conductor: (conductor = anything that lets current pass thru it)

Length of conductor, cross-sectional area and material.

- List 4 types of resistors and their particularities:

- SMT (Surface Mount Technology): gives precise resistance (Tolerances = +/- 0.02%)

- Metal Film resistors: Wide range of resistances w/ very small tolerances (Tolerance = +/- 1%)

- Carbon Composition Resistor:  cheapest (Tolerances = +/- 10% – 20%)

- Wire -wound resistors: available in low value resistances, but dissipate a lot of power. Very hot when in use

- Explain what does E12 series of resistor mean: 12 ‘Preferred Values’ of resistor, each having a tolerance range of +/-10% covers all values from 10Ω to 100Ω.

- SMT resistors: (Ex: 473 = 4.7M  5 equal # of zeros)

- Resistor Parameters: (ppm/Co)

Temp Coefficient = Resistance increase with temp  +ve temp coeff

Resistance decreases with temp  -ve temp coeff

- Why does an energized wire deflect a compass?

When electric current passes through a conductor, a magnetic field is produced around the conductor.

- Amperes law = size of magnetic field by electric current is proportional to size of that current with a constant of permeability of free space.

- What is the right-hand rule and what is it used for?

to find what way current goes (from magnetic field)

- What is a clamp-meter?

measures current by magnetic field (without touching wire)

- Skin effect:

Skin effect is the tendency of an alternating electric current (AC) to become distributed within a conductor such that the current density is largest near the surface of the conductor and decreases with greater depths in the conductor.

- What is the time constant?

Time it takes to charge a capacitor, thru a resistor, to 63.2% of the value of an applied DC voltage, or to discharge to 36.8% of its initial charge voltage.

- Faradays law:

Faraday's Law states that a changing magnetic flux on a looped conductor will produce a magnetic force that causes the electrons in the conductor to move, creating a current.

- Sources of electric power and their operating principle:

Water, fuel, wind, steam (nuclear power plant)

- Peak vs Rms:

Peak = 120 x 2

- Given signal, draw second signal time that is shifted:

Ex: v = Acos(wt + pi/4)  shifted by pi/4 (sine graph shifted by pi/4)

- Explain why omega=377 rad/sec:

Convention in Canada (Other places have 50 Hz frequencies, which is why appliances don’t work in ex Europe)

- Explain the capacitor behaviour under ac:

Capacitor will continually charge/discharge at a rate determined by the frequency of the supply

- Impedance = overall opposition of a circuit to current (Includes capacitors and inductors) (In ohms)

- Average power for an ideal capacitor:

Accepts and returns same amount of charge therefore P = 0

Transformers:

- Characteristic of an ideal transformer:

Formulas with v and I in Eq. Sheet

- Maximum power transfer:

ZL = Zs   RL = Rs  XL = -Xs

- Order of wires on the poles:

Low voltage lines (120 to 600V): below medium lines, stack of 3

Medium voltage lines (750 to 34500V): above low lines, no insulated sheath

High voltage lines: transmits power over large distances

- Why is the core made of laminations?

To reduce losses caused by eddy currents which heat up the core, meaning that electrical energy is being wasted as unwanted heat energy.

- Practical transformer:

Some current leaves the coil through air and produces an emf

- Tap transformers (how do they work and why are they popular):

Changes the voltage ratio of a transformer so that its secondary voltage stays nominal

- Slip (AC motor):

Difference between speed of electric motor magnetic field(rpm) and shaft speed (rpm). Rotor not turning = slip is max

Higher slip = higher torque

- Parameters of an electric motor:

Rated voltage:  should never be exceeded, damaging if exceeded.

Rated current: should not be exceeded for a prolonged time

- 3 Phase:

Used for high power loads (not for home use)  3 wires phase shifted 120 degrees apart

- What is the use of a starting capacitor? In what type of motors is it used?

Results in lower starting current and used for most single-phase applications (3 to 10hp)

- 3-phase electrical generator:

Generator converts mechanical power to a set of 3 AC electric currents, one from each coil. Windings are arranged such that currents vary sinusoidally at same frequency but offset peaks.

- Wye-connected: advantages and use:

If one the primary fails, secondary will still produce full voltage on all three phases.

Op-amps:

- Low-pass, high-pass and bandpass filters:

Bandpass filter: a device that passes frequencies within a certain range and rejects frequencies outside that range.

Low-pass filter: allows only everything lower than the cut-off frequency to pass through

High-pass filter: allows only everything higher than the cut-off frequency to pass through

- Effect of the feedback connection to inverting input:

to force the voltage at the inverting input to be equal to that at the noninverting input

- Physical limitations of op-amps:

Only capable to amplify signals between Vs+ and Vs-

- Effect of the feedback connection to non-inverting input:

To create hysteresis = property where a system fails to return to its original state after some cause of state change has been removed.

an op-amp with positive feedback tends to stay in whatever output state its already in. It “latches” between one of two states, saturated positive or saturated negative.

Atoms:

- Period and group significance in the periodic table:

Number on left of table (Period) = number of shells of atom

Numbers on top of table (Group) = number of valence electrons (outer most shell)

- Type of impurities added to semiconductors ad the role of impurities:

N-type semiconductors: (extra electrons negatively charged) adding an element with an extra electron (arsenic) (doping) allows arsenic to bond with silicon atoms and one electron left over for each arsenic conduct current.

P-type semiconductors: (holes positively charged) One electron missing from each neighboring covalent bond, hole where electron should be, hole assumes positive charge and attracts electrons, holes become a type of current carrier.

Diodes:

- Depletion region (region where p and n are stuck):

Positive to p and negative of battery to n  forward bias = depletion region smaller and current goes through

Positive to n and negative to p  reverse bias = depletion region bigger and current doesn’t go through

- Give 3 reasons why NAND and NOR gates are preferred:

- They are faster (less delay)

- You can make any other gate with them (AND,OR,NOT)