LABORATORY REPORT

Basic Circuits & Systems Laboratory

Course: ELEC  275 	Lab Section:

Experiment No.: 5	 	Date Performed: 2019-03-17

Experiment Title: DC MOTOR CHARACTERISTICS

Name: Mihail Mihaylov	 	ID No.:  26279228

Lab Partner Name: Laurence Ouellette & Massimo Vatistas

I certify that this submission is my original work and meets the Faculty’s Expectations of Originality

Signature: 	 	Date: 2019-03-31

Introduction

This experiment is about understanding how DC motors work. An electric motor is a machine that converts electrical energy into mechanical energy. They can be found almost everywhere, such as in home appliances, automobiles and industrial equipment. A DC Motor is mainly composed of a stator which provides a constant magnetic field, and the armature is a simple coil that rotates. The armature is connected to a DC source. When the current flows through the coil, and electromagnetic force is induced on it according to Lorentz law. This magnetic field is responsible for rotating the shaft of the motor.

Procedure:

- Make sur that the power supplies are OFF.

- Connect the circuit using the diagram in Fig. 5.3.

- Make sure the Fluke DMM is set to DC and 2000mA

- Turn the current control on the Anatek all the way to the right.

- Make sure loading screw FL is not screwed in the motor

- Connect the voltage lead from the Agilent and place it to the white terminal displayed on the motor box.

- Disconnect lead X from the box and turn on the Anatek and master power supply.

- Adjust voltage control on Anatek till it reads 7V.

- Reconnect the X lead to the terminal where it was disconnected and make sure the voltage still reads 7V.

Measurements:

- From 7V decrease it to 1V intervals and record Va, Vb, I, and Tp until 3V

- Now starting at 3.5V increase it to 1V intervals and record Va, Vb, I, and Tp until 6.5V is reached.

Mechanical Loading:

- Set voltage Va to 6V and record current I and period Tp.

- Screw in the loading screw FL till it makes contact with the axle of motor B.

- Obtain three readings at current I = 400, 450, and 500mA

Electrical Loading:

- Set voltage Va to 6V and make sure loading screw FL is not connected to the axle.

- Turn the EL switch to the right which add a 30-ohm resistance.

- Repeat steps 2,3 of Mechanical loading and record the results.

Determination of motor constant K (=Kb=KT).

From the data of Table A , calculate and enter values in the table below:

From the data in table A:

| Vb volts | w rad/s |
| --- | --- |
| 4.96 | 566 |
| 3.86 | 462 |
| 2.84 | 302.08 |
| 1.80 | 231 |
| 1.05 | 119 |
| 1.52 | 175.51 |
| 2.30 | 273.18 |
| 3.52 | 405.37 |
| 4.44 | 510.83 |

The given slop/Kb/Kt = -112.5 mN-m-sec/rad

For Torque-Speed characteristic; from table B since we know that T = Kt*I and w = pi/Tp we can see that from the graph we get:

| T mN-m; w rad/s. -44437; 442.5. -45000; 441.2. -50625; 436.33. -56250; 400.2. -52312; 462. -56250; 369.6. - |  |
| --- | --- |

- For the effective viscous friction in the unloaded condition:

B = T/w = -44377mN-m/442.5rad/s*1000 = B = -0.1003 Nm-sec/rad

- For the effective armature circuit resistance Ra: Using table A

Avg Ra to be = 267/9 = Ra = 29.67ohms

- From Table B, to find the speed regulation:

w no load = pi/7.0E-3 = 442.5

I = 500 mA: w full load = pi/8.5-3 = 369.6 rad/s

The speed regulation S = 100[442.5-369.6]/ 442.5= 16.47%

- Efficient factor table B: Va= 6 V, I= 395mA

Pin = Va*I = 6V*0.395 = 2.37 watts;

Pout = Kt*I*w = 1.54 watts;

Efficiency η = (2.37-1.54)/2.37 x 100% = 35%

In conclusion, we can say that A DC motor's speed can be controlled over a wide range, using either a variable supply voltage or by changing the strength of current in its field windings. Our results show some errors that were due to some factors out of our control such as the friction created by the heat of the motor. In general, this experiment is a success!