# Assignment 2:

Assignment Questions:

Q1. Compare between three prominent types of Digital Modulation techniques. What is SNR, What is the relation between SNR and Signal output quality? (4 marks)

- The three prominent types of Digital Modulation techniques are Amplitude Shift Keying, Frequency Shift Keying and Phase Shift Keying.

| Parameter | ASK (Amplitude Shift Keying) | FSK (Frequency Shift Keying) | PSK (Phase Shift Keying) |
| --- | --- | --- | --- |
| Function | Amplitude of carrier wave varies as per voltage of modulating signal input | Frequency of carrier wave varies as per voltage of modulating signal input | Phase of carrier wave varies as per voltage of modulating signal input |
| Carrier parameter | Frequency is kept constant | Amplitude is kept constant | Amplitude is kept constant |
| Complexity | Simple | Moderate | Very complex |
| Noise immunity | Low | High | High |
| Error Probability | High | Low | Low |
| Types | BASK | GFSK, FSK | BPSK, QPSK |

SNR (Signal to Noise Ratio) is the ratio of the signal power to the noise power. The higher the value of SNR, the greater will be the quality of the signal output.

Q2. Compare between TCP and UDP. What are the steps for three-way handshake? (4 marks)

|  | TCP (Transmission Control Protocol) | UDP (User Datagram Protocol) |
| --- | --- | --- |
| 1 | Connection Oriented Protocol | Connection-less Protocol |
| 2 | The TCP packet is called a segment | The UDP packet is called a datagram |
| 3 | Provides error control and flow control | Doesn’t provide error or flow control. |
| 4 | Reliable service for data transmission | Not reliable for data transmission |
| 5 | Relatively slow | Relatively fast |
| 6 | Keeps track of lost packets and ensures they are re-sent | Doesn’t keep track of lost packets |
| 7 | Segments Acknowledgement | No Segment Acknowledgement |
| 8 | Rearranges packets in order of how they were sent | No inherent order |
| 9 | 20 Byte header size | 8 Byte header size |

- The steps of a three-way handshake are

- SYN

- SYN and ACK

- ACK

Q3. What are the steps a router follows in order to determine an appropriate action for incoming packets? (2 marks)

- The router discards Layer 2 encapsulation so that the IP packet can be examined.

- The router examines the IP address

- The router checks the routing table for a match

- The router selects the next hop

- The router forwards the packet