Structural and Functional Neuroimaging

Early structural techniques

- Skull x-ray

  - Cannot see brain, but can see foreign objects, skull injuries

- Pneumoencephalogram

  - Puts air into CSF to photograph brain

    - Extremely painful

Computerized Axial Tomography (CT scan)

- Using radiation to image the brain

- Different density of matter absorbs radiation differently

- Safest to use in emergencies

- Bone/blood = white

- Brain = grey

- Fluid = black

Structural Magnetic Resonance Imaging (sMRI)

- Much higher resolution than CT scanning

- Hydrogen atoms in body

  - Not specifically oriented

- When subjected to magnetic field, these H atoms will orient in same direction

- Knock H atoms out of alignment with a pulse, measure rebound time

- Different density of material allows for different rebound time

** Strong magnet, dangerous. Cannot have metals in body. **

Diffusion Tensor Imaging (DTI)

- There are water molecules in the brain,

- When shaken these molecules move up and down axons further than elsewhere in the brain

- Used to map out major axon connections in the brain

- Can be color coded to see the direction

Cerebral Angiography

- Blood vessel map

- Dye injected into blood

- Can also be done during MRI (MR Angiography)

Electroencephalogram (EEG)

- Using sensors placed on various parts of the head to pick up brain waves

- Can be used to see where seizures start

- Used extensively in consciousness research (sleep cycles, etc.)

- Good for localizing damage and seeing what part of the brain acts dysfunctionally

Positron Emission Tomography (PET)

- Make radioactive substance that will particularly like to stick to a certain substance in whatever we want to scan

- PET gets its image from the radioactivity that is picked up

- ie. dementia, alzheimers, parkinsons, tumors show up differently (different oxygen patterns)

- good at looking at how networks change over time

Functional Magnetic Resonance Imaging (fMRI)

- found that magnetic properties in oxygenated blood and deoxygenated blood differed

- can be used to see where the brain is active and not active based on where the oxygenated blood is concentrated at

- good for exploring brain networks

Transcranial Magnetic Stimulation (TMS)

- stimulating brain using magnetic outside the head

- stimulates neurons, can be used to simulate damage

- used to map out homunculi in sensory/motor cortex

- also used for treatment (better than drugs for depression)

Three Challenges when studying brain injuries

- don’t have patients previously healthy brain to compare to

- strokes are very messy

- brain patterns change over time