- Requirements for the Formation of glacier:

  - Cold temperature

  - Sufficient precipitation

- Glacier mechanics

  - Internal Visco-Plastic Flow:the glacier expands outward after its own weight becomes too heavy, similar to what happens to heated cookie dough

  - Basal sliding: e glacier is stationed on a slope and pressure melts ice at the bottom, forming a thin layer of water. The loss of friction allows the glacier to "slip," or slide down the slope.

  - More ice = accumulation = will go forward

  - Less ice = ablation = will retreat

- Erosion

  - Plucking: glacier flows over a fractured bedrock surface,it loosens and lifts blocks of rock and adds into the ice. known as plucking. Land left fine to big boulders

  - Abrasion: glacier scrapes over bedrock, grinding it into rock flour.  Very fine

- Types of deposit

  - 1)Materials deposited directly by the glacier, which are known as till, and

  - 2)Sediments laid down by glacial meltwater, called stratified drift.- SORTED

    - Kame- terrace

    - Kettle- hole in ground

    - Esker-tunnel

  - Drumlin – gentle slop – ice going  in that direction ( to many contours close by)

- Engineering effect

  - Glacial-fluvial sediment: more gravel ( good for water)

  - Glacial-lacustrine sediments: more clay , less gravel , easily compress with heavy building ( good for getiing clay)

- Rivers

  - Sediment transportation :

  - Bed load: with the bed of the channel

  - Suspended load: muddy . pulp in OJ

  - Dissolved:  dissolved and dispered throught the water. Sugar in organe juice

- Depostional environements:

- braided streams

- Meandering streams

- Deposit on inside & erode on outside of bends

Flood plains

- Alluvial fans and deltas ( energy is lost , sediments are depositied) : alluvival :

  - hits the valley

  - Delta: hits ocean

- Graded streams and rivers: has the slope to maintain minimum velocity  ( just transporting no erosion or deposition)

- Habitation on floodplains.

  - Bridges & construction altering river channels.

  - Transportation along rivers.

  - Dams (hydroelectric, reservoir, navigation,

- Coastlines

  - Big waves – steep slope

  - Small wave- gentle slope

- Solutions

  - Jetty

- Groins: at 90 degree angle . they are in pairs

- Breakwater: random wall in the middle of the ocean divert the waves back

- Seawall: wall at shoreline

- Weathering

- Mechnanical wethering

  - Frost

    - Water in crack feezes and expands

  - Salt

    - Crystalization of salt in pores

  - Exfoliations

    - Fractures in rock

  - Root

  - They grow in rocks and expand

- Chemical

  - Solution / dissolution

    - Soft rock dissolve as mineral react with acid and water

  - Hydrolysis

    - React with water

  - Carbonation

    - React with carbonic acid

  - Hydrations

    - Absorbing water

  - Oxidations

  - React with oxygen

- Reasons for weathering

  - Fractures

  - Porosity of rock

  - High temperature is more likely to weather on earth’s surface

  - Movment in piece: erosion

  - Just break down : weathering

  - Ground water

  - Porosity: how much void space is there

  - Permeability: transport water

  - Hydrallic head: pressure head + elecvation head

  - Hydrallic gradient = i = h1-h2/l

  - Specific flux= q=ki

  - Volumetric flux: KiA

  - K = hydrollic conductivity

  - Velocity=discharge(q)/porosity (n)            (m/s)

  - discharge = V A

  - Aquifer: transfer water

- Confined : aquitard below and above

- Unconfined: direct to surface

  - Result in drop of water table / cone of depression

  - Aquitard: slow transfer but store water

  - Aquiclude: prevents the water movement

  - Soil

  - Top soil : Holds moisture, include organic shit . prevents erosion

  - Sub soil: holds clay and oxides. Has small rock material

  - Expansive soil = a lot of swelling clay = hold water = roller coaster roads/ cracks

  - Hydrocompaction refers to the tendency for loose, uncompacted soils that are found in some arid environments to compact under load when they are wetted

  - Collapsing soil is defined as a weakly cemented soil subject to large settlements under load and as a result of deterioration of the cementing agent in presence of water